# Lab Assignment: Non-preemptive Shortest Job First (SJF) CPU Scheduling Algorithm

## Objective:

To understand and implement the Non-preemptive Shortest Job First (SJF) CPU scheduling algorithm and compute:
- Gantt Chart
- Completion Time (CT)
- Turnaround Time (TAT)
- Waiting Time (WT)
- Average Turnaround Time
- Average Waiting Time

Reference:

Step 1: Start

Step 2: Input

- Read number of processes n

Step 3: Read Process Details

For each process i = 1 to n:

- Read Arrival Time AT[i]

- Read Burst Time BT[i]

- Set Completed[i] = false

Step 4: Initialize

- current_time = 0

- completed_count = 0

Step 5: Repeat until all processes are completed

While completed_count < n do:

- Set min_bt = ∞   // Initialize minimum burst time as very large

- Set index = -1   //  Initialize index to “no process selected”

- For each process i = 1 to n:

  - If

    - AT[i] ≤ current_time   // process has arrived

    - and Completed[i] = false  //process not finished

    - and BT[i] < min_bt   //has smaller burst time than current minimum

then:

    - min_bt = BT[i]   //update minimum burst time

    - index = I // store this process as candidate for execution

- If index = -1:

  - No process has arrived

  - current_time = current_time + 1

- Else:

  - current_time = current_time + BT[index]

  - CT[index] = current_time

  - TAT[index] = CT[index] - AT[index]

  - WT[index] = TAT[index] - BT[index]

  - Completed[index] = true

  - completed_count++

Step 6: Compute Averages

- Average Turnaround Time = ΣTAT / n

- Average Waiting Time = ΣWT / n

Step 7: Display Results

Step 8: Stop